CPE301 – SPRING 2019

# Design Assignment 2AT1

Student Name: Billy Maddex

Student #: 2000928390

Student Email: maddex@unlv.nevada.edu

Primary Github address: https://github.com/billymaddex/fluffy-chainsaw

Directory: DA2A/T1

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB-XMINI

Multi-Function Arduino Module

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

;

; DA2AT1.asm

;

; Created: 9/29/2019 3:15:15 PM

; Author : Billy

;

;; Set GPIO ports

;; Set DDR B,C,D to known initial value

LDI		R25, 0x00

OUT		DDRB, R25

OUT		DDRC, R25

OUT		DDRD, R25

;; set PORT B,C,D to know initial value

LDI		R25, 0xFF

OUT		PORTB, R25

OUT		PORTC, R25

OUT		PORTD, R25

;; set PORTB 2 and 3 to output

SBI		DDRB, 3

SBI		DDRB, 2

;; set PINC 3 to input

CBI		DDRC, 3

MAIN_LOOP:

;; LED pulse

;; turn the light on

CBI		PORTB, 3

;; call delay at 250ms

LDI		R17, HIGH(250)

LDI		R16, LOW(250)

CALL		DELAY

;; turn the light off

SBI		PORTB, 3

;; call delay at 375ms

LDI		R17, HIGH(375)

LDI		R16, LOW(375)

CALL		DELAY

;; if switch is not pressed, repeat the loop

SBIC		PINC, 3

RJMP		MAIN_LOOP

;; otherwise, flash the other light

;; turn the light on

CBI		PORTB, 2

;; call delay at 1333ms

LDI		R17, HIGH(1333)

LDI		R16, LOW(1333)

CALL		DELAY

;; turn the light off

SBI		PORTB, 2

;; repeat the loop

RJMP	MAIN_LOOP

;; simple delay function

;; takes input as 16-bit int in R17:R16

;; delay is supposed to be in ms

DELAY:

LDI		R20, 0

MOV		R22, R17

MOV		R21, R16

L3:

LDI		R23, 0X2F

L2:

LDI		R24, 0X70

NOP

L1:

DEC		R24

BRNE		L1

DEC		R23

BRNE		L2

SUBI		R21, 1

SBCI		R22, 0

BRNE		L3

CP		R21, R20

BRNE		L3

RET

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/jWZDmGci-QE

- GITHUB LINK OF THIS DA

https://github.com/billymaddex/fluffy-chainsaw/tree/master/DesignAssignments/DA2A/T1

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Billy Maddex