CPE301 – SPRING 2019

# Design Assignment 2BT2

Student Name: Billy Maddex

Student #: 2000928390

Student Email: maddex@unlv.nevada.edu

Primary Github address: https://github.com/billymaddex/fluffy-chainsaw

Directory: DA2B/T2

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB-XMINI

Multi-Function Arduino Module

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA2AT2.c

*

* Created: 9/29/2019 7:42:05 PM

* Author : Billy

*/

#define F_CPU 1600000UL

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

// Set DDR B,C,D to known initial value

DDRB = 0x00;

DDRC = 0x00;

DDRD = 0x00;

// set PORT B,C,D to know initial value

PORTB = 0xFF;

PORTC = 0xFF;

PORTD = 0xFF;

// set PORTB 2 and 3 to output

DDRB |= (1 << 3);

DDRB |= (1 << 2);

// set PINC 3 to input

DDRC &= ~(1 << 3);

// LED pulse

while (1)

{

// turn the light on

PORTB &= ~(1 << 3);

// call delay

_delay_ms(2500);

// turn the light off

PORTB |= (1 << 3);

// call delay

_delay_ms(3750);

// if switch is pressed, flash the other light

if (~PINC & (1 << 3))

{

// turn the light on

PORTB &= ~(1 << 2);

// call delay

_delay_ms(13330);

// turn the light off

PORTB |= (1 << 2);

}

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

/*

* DA2BT2.c

*

* Created: 10/5/2019 2:28:51 PM

* Author : Billy

*/

#define F_CPU 1600000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

// long flash on secondary LED

int light2 (void)

{

// turn the light on

PORTB &= ~(1 << 2);

// call delay

_delay_ms(13330);

// turn the light off

PORTB |= (1 << 2);

return 0;

}

// external interrupt handler

ISR (INT0_vect)

{

if (~PIND & (1 << 2))

{

light2();

}

}

// pin change interrupt handler

ISR (PCINT1_vect)

{

if (~PINC & (1 << 3))

{

light2();

}

}

int main(void)

{

// Set DDR B,C,D to known initial value

DDRB = 0x00;

DDRC = 0x00;

DDRD = 0x00;

// set PORT B,C,D to know initial value

PORTB = 0xFF;

PORTC = 0xFF;

PORTD = 0xFF;

// set PORTB 2 and 3 to output

DDRB |= (1 << 3);

DDRB |= (1 << 2);

// set PINC 3 to input

DDRC &= ~(1 << 3);

// configure interrupt handling

// enable INT0, falling edge trigger

EIMSK = (1 << INT0);

EICRA = 0x02;

// enable PCINT11 (PINC.3)

PCMSK1 = 0x08;

PCICR = (1 << PCIE1);

// enable global interrupt

sei();

// LED pulse

while (1)

{

// turn the light on

PORTB &= ~(1 << 3);

// call delay

_delay_ms(2500);

// turn the light off

PORTB |= (1 << 3);

// call delay

_delay_ms(3750);

// if switch is pressed, flash the other light

/*if (~PINC & (1 << 3))

{

light2();

}*/

}

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/sffTvS2AUXs

- GITHUB LINK OF THIS DA

https://github.com/billymaddex/fluffy-chainsaw/tree/master/DesignAssignments/DA2B/T2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Billy Maddex