CPE301 – SPRING 2019

# Design Assignment 2CT3

Student Name: Billy Maddex

Student #: 2000928390

Student Email: maddex@unlv.nevada.edu

Primary Github address: https://github.com/billymaddex/fluffy-chainsaw

Directory: DA2C/DA2CT3

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB-XMINI

Multi-Function Arduino Module

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 2/B

/*

* DA2BT2.c

*

* Created: 10/5/2019 2:28:51 PM

* Author : Billy

*/

#define F_CPU 1600000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

// long flash on secondary LED

int light2 (void)

{

// turn the light on

PORTB &= ~(1 << 2);

// call delay

_delay_ms(13330);

// turn the light off

PORTB |= (1 << 2);

return 0;

}

// external interrupt handler

ISR (INT0_vect)

{

if (~PIND & (1 << 2))

{

light2();

}

}

// pin change interrupt handler

ISR (PCINT1_vect)

{

if (~PINC & (1 << 3))

{

light2();

}

}

int main(void)

{

// Set DDR B,C,D to known initial value

DDRB = 0x00;

DDRC = 0x00;

DDRD = 0x00;

// set PORT B,C,D to know initial value

PORTB = 0xFF;

PORTC = 0xFF;

PORTD = 0xFF;

// set PORTB 2 and 3 to output

DDRB |= (1 << 3);

DDRB |= (1 << 2);

// set PINC 3 to input

DDRC &= ~(1 << 3);

// configure interrupt handling

// enable INT0, falling edge trigger

EIMSK = (1 << INT0);

EICRA = 0x02;

// enable PCINT11 (PINC.3)

PCMSK1 = 0x08;

PCICR = (1 << PCIE1);

// enable global interrupt

sei();

// LED pulse

while (1)

{

// turn the light on

PORTB &= ~(1 << 3);

// call delay

_delay_ms(2500);

// turn the light off

PORTB |= (1 << 3);

// call delay

_delay_ms(3750);

// if switch is pressed, flash the other light

/*if (~PINC & (1 << 3))

{

light2();

}*/

}

}

- DEVELOPED MODIFIED CODE OF TASK 3/C from TASK 2/B

/*

* DA2CT3.c

*

* Created: 10/12/2019 1:35:55 PM

* Author : Billy

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

// global constants and variables

const int COMP = 249;

const int ON1 = 250;

const int OFF1 = 375;

const int ON2 = 1333;

int clock1, clock2, led3;

// long flash on secondary LED

int light2 (void)

{

// turn the light on

PORTB &= ~(1 << 2);

// set clock 2 to CTC operation

TCCR2A = (1 << WGM21);

// COMPA = 249

OCR2A = COMP;

// set the clock value

clock2 = ON2;

// start clock 2 with prescalar = 64

TCCR2B |= (1 << CS22);

return 0;

}

// external interrupt handler

ISR (INT0_vect)

{

if (~PIND & (1 << 2)) light2();

}

// pin change interrupt handler

ISR (PCINT1_vect)

{

if (~PINC & (1 << 3)) light2();

}

// timer0 OVF interrupt handler

ISR (TIMER0_COMPA_vect)

{

// decrement the clock counter and reset the timer

clock1--;

TIFR0 |= (1 << OCF0A);

if (clock1 == 0)

{

// if LED3 is on

if (led3)

{

// turn LED3 off

PORTB |= (1 << 3);

led3 = 0;

// set the off clock1 value

clock1 = OFF1;

}

// if LED3 in off

else

{

// turn LED3 on

PORTB &= ~(1 << 3);

led3 = 1;

// set the on clock1 value

clock1 = ON1;

}

}

}

// timer2 OVF interrupt handler

ISR (TIMER2_COMPA_vect)

{

// decrement the clock counter and reset the timer

clock2--;

TIFR2 |= (1 << OCF2A);

if (clock2 == 0)

{

// turn LED off

PORTB |= (1 << 2);

// stop timer 2

TCCR2B &= ~(1 << CS22);

}

}

int main(void)

{

// Set DDR B,C,D to known initial value

DDRB = 0x00;

DDRC = 0x00;

DDRD = 0x00;

// set PORT B,C,D to know initial value

PORTB = 0xFF;

PORTC = 0xFF;

PORTD = 0xFF;

// set PORTB 2 and 3 to output

DDRB |= (1 << 3);

DDRB |= (1 << 2);

// set PINC 3 to input

DDRC &= ~(1 << 3);

// configure interrupt handling

// enable INT0, falling edge trigger

EIMSK = (1 << INT0);

EICRA = 0x02;

// enable PCINT11 (PINC.3)

PCMSK1 = 0x08;

PCICR = (1 << PCIE1);

// enable TIMER0 COMPA interrupt

TIMSK0 |= (1 << OCIE0A);

// enable TIMER2 COMPA interrupt

TIMSK2 |= (1 << OCIE2A);

// LED pulse

// set clock 1 to CTC operation

TCCR0A |= (1 << WGM01);

// COMPA = 249

OCR0A = COMP;

// turn the light on

PORTB &= ~(1 << 3);

led3 = 1;

// set the on clock1 value

clock1 = ON1;

// start clock 1 with prescalar = 64

TCCR0B |= (1 << CS01) | (1 << CS00);

// enable global interrupt

sei();

// loop forever

while (1);

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/JV2nWGm-CmA

- GITHUB LINK OF THIS DA

https://github.com/billymaddex/fluffy-chainsaw/tree/master/DesignAssignments/DA2C/DA2CT3

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Billy Maddex