CPE301 – SPRING 2019

# Design Assignment 3A

Student Name: Billy Maddex

Student #: 2000928390

Student Email: maddex@unlv.nevada.edu

Primary Github address: https://github.com/billymaddex/fluffy-chainsaw

Directory: DA3A

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB-XMINI

Multi-Function Arduino Module

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 3A

/*

* DA3A.c

*

* Created: 10/19/2019 9:26:49 PM

* Author : Billy

*/

// 16MHz CPU clock

#define F_CPU 16000000UL

// 9600 baud rate setting

#define UBRR_9600 103

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/atomic.h>

#include <stdlib.h>

#include <stdio.h>

// global constants and variables

// terminal output variables

int ival;

float fval;

char sendme[80];

// terminal output function declarations

void uart_init(unsigned int ubrr);

void uart_send_string(char *data);

// TIMER1 CTC interrupt handler

ISR (TIMER1_COMPA_vect	)

{

// get a random integer

ival = rand();

fval = (float) ival / rand();

// print to the terminal: a string, a random int, a floating point

ATOMIC_BLOCK(ATOMIC_FORCEON)

{

snprintf(sendme, sizeof(sendme), "A string every second!\nA random integer: %d\nA floating point: %f\r\n\r\n", ival, fval);

uart_send_string(sendme);

}

}

int main(void)

{

// configure terminal and test output

uart_init(UBRR_9600);

// configure interrupt handling

// enable TIMER1 COMPA interrupt

TIMSK1 |= (1 << OCIE1A);

// set TIMER1 to CTC operations

TCCR1B |= (1 << WGM12);

// TIMER1 COMPA = X

OCR1A = 15620;

// start TIMER1 with prescalar = 1024

TCCR1B |= (1 << CS12) | (1 << CS10);

// enable global interrupt

sei();

// loop forever

while (1);

}

// configure terminal output

void uart_init(unsigned int ubrr)

{

// set buad rate to 9600

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

// set terminal to output

UCSR0B = (1 << TXEN0);

// set ouput mode

UCSR0C = (1 << UCSZ01) | (1 << UCSZ00);

}

void uart_send_string(char *data)

{

while((*data != '\0'))

{

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

- DEVELOPED MODIFIED CODE OF TASK X/X from TASK X/X

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/JtFsujSQ8sY

- GITHUB LINK OF THIS DA

https://github.com/billymaddex/fluffy-chainsaw/tree/master/DesignAssignments/DA3A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Billy Maddex