CPE301 – SPRING 2019

# Design Assignment 4A

Student Name: Billy Maddex

Student #: 2000928390

Student Email: maddex@unlv.nevada.edu

Primary Github address: https://github.com/billymaddex/fluffy-chainsaw

Directory: DA4A

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB-XMINI

Breadboard Arduino Module

1K Ohm Potentiometer

LED (to show PWM duty cycle output)

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 3A

/*

* DA4A.c

*

* Created: 11/11/2019 8:45:32 AM

* Author : Billy

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

// pin change vector

// toggle PWM output signal on each button press

ISR(PCINT0_vect)

{

if(~PINB & (1 << 7))

{

DDRD ^= (1 << DDRB3);

}

}

// ADC conversion complete vector

// update PWM duty cycle on each conversion

ISR(ADC_vect)

{

unsigned int voltage = ADCH;

OCR2B = (voltage * 242) / 255;

}

// configure pin change interrupt for PINB7

void button_init()

{

PCMSK0 = (1 << PCINT7);

PCICR = (1 << PCIE0);

}

// configure TIMER2 for non-inverting fast PWM, OCR2B/PIND3 output,prescalar 64

void PWM_init()

{

TCCR2B |= (0 << WGM22) | (1 << CS22) | (0 << CS21) | (0 << CS20);

TCCR2A |= (1 << COM2B1) | (0 << COM2B0) | (1 << WGM21) | (1 << WGM20);

}

// enable ADC on PINC5, free running, interrupt enable

void ADC_init()

{

ADMUX =

// Reference Selection Bits

// AVcc with external capacitor at AREF

(0 << REFS1) |

(1 << REFS0) |

// ADC Left Adjust Result

(1 << ADLAR) |

// Analog Channel Selection Bits

// ADC5 (PC5 PIN28)

(0 << MUX3) |

(1 << MUX2) |

(0 << MUX1) |

(1 << MUX0);

ADCSRA =

// ADC ENable

(1 << ADEN) |

// ADC Start Conversion

(1 << ADSC) |

// ADC Auto Trigger Enable

(1 << ADATE) |

// ADC Interrupt Flag

(0 << ADIF) |

// ADC Interrupt Enable

(1 << ADIE) |

// ADC Prescaler Select Bits

// prescalar = 128

(1 << ADPS2) |

(1 << ADPS1) |

(1 << ADPS0);

ADCSRB =

// disable analog comparator

(0 << ACME) |

// enable free running mode

(0 << ADTS2) |

(0 << ADTS1) |

(0 << ADTS0);

}

int main(void)

{

// initialize input buttons

button_init();

// initialize PWM

PWM_init();

// initialize ADC

ADC_init();

// enable those interrupts

sei();

// the usual infinite loop

while (1);

}

- DEVELOPED MODIFIED CODE OF TASK B from TASK A

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/kstyw8gMm5U

- GITHUB LINK OF THIS DA

https://github.com/billymaddex/fluffy-chainsaw/tree/master/DesignAssignments/DA4A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Billy Maddex