hCPE301 – SPRING 2019

# Design Assignment 4B

Student Name: Billy Maddex

Student #: 2000928390

Student Email: maddex@unlv.nevada.edu

Primary Github address: https://github.com/billymaddex/fluffy-chainsaw

Directory: DA4B

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB-XMINI

Breadboard Arduino Module

1K Ohm Potentiometer

Stepper Motor and Driver

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 4BT1

/*

* DA4B.c

*

* Created: 12/3/2019 11:42:58 PM

* Author : Billy

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

// global variable to control motor step

int step = 0;

// ADC conversion complete vector

// update CTC COMPA on each conversion

ISR(ADC_vect)

{

unsigned int voltage = ADCH;

// TIMER1 COMPA = X

OCR1A = (( (float) voltage / 255.0) * 1480) + 80;

}

// TIMER1 CTC interrupt handler

ISR(TIMER1_COMPA_vect)

{

// check step variable to move through full step sequence

if (step == 0)

{

PORTD = 0x09;

}

else if (step == 1)

{

PORTD = 0x03;

}

else if (step == 2)

{

PORTD = 0x06;

}

else if (step == 3)

{

PORTD = 0x0C;

}

// increment step

step++;

// reset step when greater than 3

if (step > 3)

{

step = 0;

}

}

// configure TIMER1 for CTC mode with interrupt enabled, prescalar 1024

void CTC_init()

{

// configure interrupt handling

// enable TIMER1 COMPA interrupt

TIMSK1 |= (1 << OCIE1A);

// set safe initial value for COMPA

OCR1A = 1560;

// set TIMER1 to CTC operations

TCCR1B |= (1 << WGM12);

// start TIMER1 with prescalar = 1024

TCCR1B |= (1 << CS12) | (1 << CS10);

}

// enable ADC on PINC5, free running, interrupt enable

void ADC_init()

{

ADMUX =

// Reference Selection Bits

// AVcc with external capacitor at AREF

(0 << REFS1) |

(1 << REFS0) |

// ADC Left Adjust Result

(1 << ADLAR) |

// Analog Channel Selection Bits

// ADC0 (PC0)

(0 << MUX3) |

(0 << MUX2) |

(0 << MUX1) |

(0 << MUX0);

ADCSRA =

// ADC ENable

(1 << ADEN) |

// ADC Start Conversion

(1 << ADSC) |

// ADC Auto Trigger Enable

(1 << ADATE) |

// ADC Interrupt Flag

(0 << ADIF) |

// ADC Interrupt Enable

(1 << ADIE) |

// ADC Prescaler Select Bits

// prescalar = 128

(1 << ADPS2) |

(1 << ADPS1) |

(1 << ADPS0);

ADCSRB =

// disable analog comparator

(0 << ACME) |

// enable free running mode

(0 << ADTS2) |

(0 << ADTS1) |

(0 << ADTS0);

}

int main(void)

{

// initialize ADC

ADC_init();

// initialize TIMER1

CTC_init();

// enable those interrupts

sei();

// set PORTD 0,1,2,3 to output

DDRD = 0x0F;

// the usual infinite loop

while (1);

}

- DEVELOPED MODIFIED CODE OF TASK 4BT2

/*

* DA4BT2.c

*

* Created: 12/4/2019 10:59:47 AM

* Author : Billy

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

// ADC conversion complete vector

// update CTC COMPA on each conversion

ISR(ADC_vect)

{

unsigned int voltage = ADCH;

// TIMER2 COMPB = X

// uses top 4 bits of ADC to select value from 16 to 32

OCR2B = (voltage >> 4) + 16;

}

// configure TIMER2 for PWM mode with with 50 Hz frequency. prescalar 1024, COMP2A 156

void PWM_init()

{

// set PORTD3 to output for PWM

DDRD = (1 << DDRD3);

OCR2A = 156;

TCCR2B |= (1 << WGM22) | (1 << CS22) | (1 << CS21) | (1 << CS20);

TCCR2A |= (1 << COM2A1) | (0 << COM2A0) | (1 << COM2B1) | (0 << COM2B0) | (1 << WGM21) | (1 << WGM20);

}

// enable ADC on PINC5, free running, interrupt enable

void ADC_init()

{

ADMUX =

// Reference Selection Bits

// AVcc with external capacitor at AREF

(0 << REFS1) |

(1 << REFS0) |

// ADC Left Adjust Result

(1 << ADLAR) |

// Analog Channel Selection Bits

// ADC5 (PC5 PIN28)

(0 << MUX3) |

(1 << MUX2) |

(0 << MUX1) |

(1 << MUX0);

ADCSRA =

// ADC ENable

(1 << ADEN) |

// ADC Start Conversion

(1 << ADSC) |

// ADC Auto Trigger Enable

(1 << ADATE) |

// ADC Interrupt Flag

(0 << ADIF) |

// ADC Interrupt Enable

(1 << ADIE) |

// ADC Prescaler Select Bits

// prescalar = 128

(1 << ADPS2) |

(1 << ADPS1) |

(1 << ADPS0);

ADCSRB =

// disable analog comparator

(0 << ACME) |

// enable free running mode

(0 << ADTS2) |

(0 << ADTS1) |

(0 << ADTS0);

}

int main(void)

{

// initialize ADC

ADC_init();

// enable those interrupts

sei();

// initialize PWM TIMER2

PWM_init();

// the usual infinite loop

while (1);

}

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Task 1

Task 2

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/SV1M25oX3Rw

- GITHUB LINK OF THIS DA

https://github.com/billymaddex/fluffy-chainsaw/tree/master/DesignAssignments/DA4B

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Billy Maddex