hCPE301 – SPRING 2019

# Design Assignment 6

Student Name: Billy Maddex

Student #: 2000928390

Student Email: maddex@unlv.nevada.edu

Primary Github address: https://github.com/billymaddex/fluffy-chainsaw

Directory: DA6

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB-XMINI

Breadboard Arduino Module

GY-521 MPU6050

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1

/*

* DA6.c

*

* Created: 12/8/2019 10:37:43 PM

* Author : Billy

*/

// 16MHz CPU clock

#define F_CPU 16000000UL

// 115200 baud rate setting

#define URRR_115200 8

// 9600 baud rate setting

#define UBRR_9600 103

#include <avr/io.h>

#include <util/atomic.h>

#include <util/delay.h>

#include <stdio.h>

#include "i2cmaster.h"

#include "MPU6050_def.h"

// MPU6050 read and write addresses

#define MPU6050_WRITE 0xD0

#define MPU6050_READ 0xD1

// terminal output function declarations

void uart_init(unsigned int ubrr);

void uart_send_string(char *data);

// MPU6050 function declarations

void init_MPU6050(void);

float get_reading(unsigned char data);

int main(void)

{

// configure terminal

uart_init(URRR_115200);

// initialize i2c

i2c_init();

// initialize MPU6050

init_MPU6050();

// variables to store data from MPU6050

float acl_x, acl_y, acl_z;

float gyr_x, gyr_y, gyr_z;

// output string for terminal display

char out_string[200];

// loop forever collecting and formatting the data, output to terminal

while (1)

{

// output every 1 second

_delay_ms(1000);

// update all variables with readings from the peripheral

acl_x = get_reading(ACCEL_XOUT_H);

_delay_ms(200);

acl_y = get_reading(ACCEL_YOUT_H);

_delay_ms(200);

acl_z = get_reading(ACCEL_ZOUT_H);

_delay_ms(200);

gyr_x = get_reading(GYRO_XOUT_H);

_delay_ms(200);

gyr_y = get_reading(GYRO_YOUT_H);

_delay_ms(200);

gyr_z = get_reading(GYRO_ZOUT_H);

// generate output string for acl_x and send to terminal

snprintf(out_string, sizeof(out_string), "Accel X:\t%.0f\r\nAccel Y:\t%.0f\r\nAccel Z:\t%.0f\r\nGyro X:\t\t%.0f\r\nGyro Y:\t\t%.0f\r\nGyro Z:\t\t%.0f\r\n\r\n", acl_x, acl_y, acl_z, gyr_x, gyr_y, gyr_z);

// send output string to terminal

ATOMIC_BLOCK(ATOMIC_FORCEON)

{

uart_send_string(out_string);

}

}

}

// configure terminal output

void uart_init(unsigned int ubrr)

{

// set buad rate to 9600

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

// set terminal to output

UCSR0B = (1 << TXEN0);

// set output mode

UCSR0C = (1 << UCSZ01) | (1 << UCSZ00);

}

void uart_send_string(char *data)

{

while((*data != '\0'))

{

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

// get reading for MPU6050 register specified by pointer 'data'

float get_reading(unsigned char data)

{

float value;

i2c_start(MPU6050_WRITE);

i2c_write(data);

i2c_stop();

i2c_start(MPU6050_READ);

//value = (((int)i2c_readAck()<<8) | (int)i2c_readAck());

int high = ((int)i2c_readAck()<<8);

value = high | (int)i2c_readNak();

i2c_stop();

return value;

}

// initialize MPU6050 peripheral

void init_MPU6050(void)

{

// power up delay

_delay_ms(150);

// reset all registers in the peripheral

i2c_start(MPU6050_WRITE);

i2c_write(PWR_MGMT_1);

i2c_write(0x80);

i2c_stop();

// reset delay

_delay_ms(900);

// set sample rate divider to 8, sample rate = 8kHz

i2c_start(MPU6050_WRITE);

i2c_write(SMPLRT_DIV);

i2c_write(0x07);

i2c_stop();

// set clock source to internal 8MHz and disable temp sensor

i2c_start(MPU6050_WRITE);

i2c_write(PWR_MGMT_1);

i2c_write(0x08);

i2c_stop();

// DLPF configured for 8kHz

i2c_start(MPU6050_WRITE);

i2c_write(CONFIG);

i2c_write(0x00);

i2c_stop();

// gyro configuration, set to 500 degrees per second

i2c_start(MPU6050_WRITE);

i2c_write(GYRO_CONFIG);

i2c_write(0x08);

i2c_stop();

// accelerometer configuration, set to 8g

i2c_start(MPU6050_WRITE);

i2c_write(ACCEL_CONFIG);

i2c_write(0x10);

i2c_stop();

// enable the data ready interrupt

//i2c_start(MPU6050_WRITE);

//i2c_write(INT_ENABLE);

//i2c_write(0x01);

//i2c_stop();

}

- DEVELOPED MODIFIED CODE OF TASK 2

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/8ptLOUYCFF4

- GITHUB LINK OF THIS DA

https://github.com/billymaddex/fluffy-chainsaw/tree/master/DesignAssignments/DA6

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Billy Maddex