CPE301 – SPRING 2019

# MIDTERM I

Student Name: Billy Maddex

Student #: 2000928390

Student Email: maddex@unlv.nevada.edu

Primary Github address: maddex@unlv.nevada.edu

Directory: Midterm

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/Midterm, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328PB-XMINI

LM35 Temperature Sensor → PINC5

ESP01 WiFi Module → PIND0/PIND1

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1

/*

* Midterm.c

*

* Created: 10/27/2019 12:50:46 PM

* Author : Billy

*/

// 16MHz CPU clock

#define F_CPU 16000000UL

// 115200 baud rate setting

#define URRR_115200 8

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/atomic.h>

#include <stdlib.h>

#include <stdio.h>

#include <util/delay.h>

// 15s timer variable, interval is 14 because we count from 0

const int INTERVAL = 14;

int transmit_time;

// terminal setup and output function declarations

void uart_init(unsigned int ubrr);

void uart_send_string(char *data);

// ADC function declarations

void adc_init(void);

// ESP01 setup declaration

void esp01_init(void);

// function to transmit temperature value

void xmit_temp(float temp);

// TIMER1 CTC interrupt handler

ISR(TIMER1_COMPA_vect)

{

// when transmit time is up, do something

if (transmit_time-- == 0)

{

// start ADC conversion

ADCSRA |= (1 << ADSC);

// reset transmit_time

transmit_time = INTERVAL;

}

}

ISR(ADC_vect)

{

// collect and compute ADC value

// value stored in ADC

float cel = ((5.0 * ADC) / 1000.0) + 4;

float fahr = (1.8 * cel) + 32;

// transmit the temp data

xmit_temp(fahr);

}

int main(void)

{

// configure terminal

uart_init(URRR_115200);

// configure and start ADC

adc_init();

// configure ESP01 and connect to WiFi

esp01_init();

// initialize transmit_time

transmit_time = INTERVAL;

// configure interrupt handling

// enable TIMER1 COMPA interrupt

TIMSK1 |= (1 << OCIE1A);

// set TIMER1 to CTC operations

TCCR1B |= (1 << WGM12);

// TIMER1 COMPA = X

OCR1A = 15620;

// start TIMER1 with prescalar = 1024

TCCR1B |= (1 << CS12) | (1 << CS10);

// enable global interrupt

sei();

// loop forever

while (1);

}

// configure terminal output

void uart_init(unsigned int ubrr)

{

// set baud rate

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

// set terminal to input and output

UCSR0B = (1 << TXEN0) | (1 << RXEN0);

// set output mode 8-bit data

//UCSR0C = (1 << UCSZ01) | (1 << UCSZ00);

UCSR0C =

// async operation

(0 << UMSEL01) |

(0 << UMSEL00) |

// Parity disabled

(0 << UPM01) |

(0 << UPM00) |

// stop Bits = 1bit

(0 << USBS0) |

// 8 Data bits

(1 << UCSZ01) |

(1 << UCSZ00) |

// for Synch Mode only clock polarity

(0 << UCPOL0);

}

void uart_send_string(char *data)

{

// send until the string is done

while((*data != '\0'))

{

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = *data;

data++;

}

// required delay between UART commands

_delay_ms(1800);

}

// configure ADC input

void adc_init (void)

{

// set PORTC to input for ADC

DDRC = 0x00;

ADMUX =

// Reference Selection Bits

// AVcc with external capacitor at AREF

(0 << REFS1) |

(1 << REFS0) |

// ADC Left Adjust Result

(1 << ADLAR) |

// Analog Channel Selection Bits

// ADC5 (PC5 PIN28)

(0 << MUX3) |

(1 << MUX2) |

(0 << MUX1) |

(1 << MUX0);

ADCSRA =

// ADC ENable

(1 << ADEN) |

// ADC Start Conversion

(0 << ADSC) |

// ADC Auto Trigger Enable

(0 << ADATE) |

// ADC Interrupt Flag

(0 << ADIF) |

// ADC Interrupt Enable

(1 << ADIE) |

// ADC Prescaler Select Bits

// prescalar = 128

(1 << ADPS2) |

(1 << ADPS1) |

(1 << ADPS0);

}

void xmit_temp(float temp)

{

// output strings

char send[20];

char report[80];

int report_len = 0;

// generate output string

snprintf(report, sizeof(report), "GET /update?api_key=TNTAKRCSHOJW0ANL&field1=%.2f\r\n\r\n", temp);

// get number of bytes to send

while (report[report_len] != '\0')

{

report_len++;

}

// generate SEND AT string based on byte size of report

snprintf(send, sizeof(send), "AT+CIPSEND=%d\r\n", report_len);

// output to terminal

ATOMIC_BLOCK(ATOMIC_FORCEON)

{

uart_send_string("AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n");

uart_send_string(send);

uart_send_string(report);

}

}

// configure ESP01 and connect to WiFi

void esp01_init(void)

{

uart_send_string("AT\r\n");

uart_send_string("AT+CWMODE=1\r\n");

uart_send_string("AT+CIPSTA=\"10.7.7.22\"\r\n");

uart_send_string("AT+CWJAP=\"xxxxxxxx\",\"xxxxxxxx\"\r\n");

uart_send_string("AT+CIPMUX=0\r\n");

}

- DEVELOPED MODIFIED CODE OF TASK 2 from TASK 1

// no modified code

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/upMDbBFYtDk

https://thingspeak.com/channels/895826

- GITHUB LINK OF THIS DA

https://github.com/billymaddex/fluffy-chainsaw/tree/master/Midterm

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Billy Maddex